# SR20DET ECU S-13 PINOUT

| ECU PINS | Description of use | Wire color |
| --- | --- | --- |
| 1 | Coil Pack 1 Output (Via power transistor Unit) | Red White |
| 2 | Coil Pack 2 Output (Via power transistor Unit) | Red Yellow |
| 3 | Tachometer signal output | Yellow Red |
| 4 | ECCS Relay | Red Black |
| 5 | Ignition Pulse Monitor (For Trip Comp ?) |  |
| 6 | Ignition Signal Ground | Black |
| 7 | RX data (Signal from Nissan Consult hand terminal) | Green Black |
| 8 | Coil Pack 3 Output (Via power transistor Unit) | Red White |
| 9 | Coil Pack 4 Output (Via power transistor Unit) | Red White |
| 10 | - |  |
| 11 | - |  |
| 12 | A/T signal DT3 (autotrans gear select ?) |  |
| 13 | Ignition Signal Ground | Black |
| 14 | CLK clock (timing signal) | Green White |
| 15 | TX data (signal to Nissan Consult hand terminal) | Green |
| 16 | MAFS signal input ( Mass Air Flow Sensor ) | White |
| 17 | MAFS ground | Black |
| 18 | Water Temp Sensor input | Blue Brown |
| 19 | Oxyen Sensor input (O2) | White |
| 20 | TPS input (throttle postion sensor) | White |
| 21 | Sensor ground | Black |
| 22 | CAS 180deg signal (Crank angle sensor) | White |
| 23 | CHK check (check start) ? | Green Red |
| 24 | Monitor check Lamp (red) (fault code lamp output) |  |
| 25 | Exhaust over temp warning lamp output | Orange |
| 26 | Exhaust temp sensor inout (Behind the Cat) | Blue Yellow |
| 27 | Detonation sensorinput (knock sensor) | White |
| 28 | TPS for A/T (kick down switch) |  |
| 29 | Sensor ground | Black |
| 30 | CAS 180deg signal (crank angle sensor) | White |
| 31 | CAS 1deg signal (crank angle sensor) | Black |
| 32 | Speed sensor input | Yellow Green |
| 33 | - |  |
| 34 | Start signal (from ignition switch) | Orange |
| 35 | A/T neutral switch input | Green Orange |
| 36 | IGN (ignition ON, from ignition switch) | Black Red |
| 37 | TPS power output (power to throttle position sensor) | Light Green Red |
| 38 | ECCS power | Black White |
| 39 | ECCS ground | Black |

| ECU PINS | Description of use | Wire color |
| --- | --- | --- |
| 40 | CAS 1 deg signal (crank angle sensor) | Red Black |
| 41 | A/C on input (Air Con ON from A/C panel) | Blue Green |
| 42 | A/T signal DT1 (auto trans gear select ?) |  |
| 43 | Power steering switch (speed proportional control output) | Purple White |
| 44 | A/T signal DT1 (auto trans gear select ?) |  |
| 45 | Ambient air tamp sensor input | Blue White |
| 46 | ECCS backup power | Red |
| 47 | ECCS power | Black White |
| 48 | ECCS ground | Black |
| 101 | Injector No. 1 drive output | White Black |
| 102 | EAI control solenoid output (egr valve control ?) | Orange Black |
| 103 | Injector No.3 drive output | Green Black |
| 104 | Fuel pump relay output | Black Purple |
| 105 | Fuel pressure control module | Red White |
| 106 | A/C on output (Air conditioner on relay signal) | Green Black |
| 107 | Injector ground | Black |
| 108 | Injector ground | Black |
| 109 | Reverse electrical flow return circuit (alternator fail input) | Red |
| 110 | Injector No. 2 drive output | Yellow Black |
| 111 | Turbo boost pressure control solenoid (over boost cut ?) | Orange Black |
| 112 | Injector No. 4 drive output | Blue Black |
| 113 | AAC valve control (Auxiliary Air Control, Idle speed control) | Light Blue |
| 114 | - |  |
| 115 | - |  |
| 116 | Injector ground | Black |